2012 q9

Define resistance.

The resistance of an object within an electrical circuit is defined as the ratio of the voltage across it to the current flowing through it, i.e. .

- Two resistors of resistance R1 and R2 are connected in series. Derive an expression for the effective resistance of the two resistors in terms of R1 and R2.

### Resistors in series

R1

a

b

R2

This would be an example of two resistors connected in series, carrying a current, I.

For this set-up, the total voltage drop from a to b is given by

Where V1 is the voltage drop across R1, V2 is the voltage drop across R2.

Following Ohm's Law we can say:

and,  dividing the equation by I,

- Two 4 Ω resistors are connected in parallel. 
Draw a circuit diagram to show how another 4 Ω resistor could be arranged with these two resistors to give an effective resistance of 6 Ω.

4Ω

4Ω

4

4Ω

- A fuse is a resistor used as a safety device in a circuit. How does a fuse operate?

A fuse is simply a short piece of relatively weak wire, chosen so that if a current above a set value should flow through it, it will overheat and break. As this will break the circuit, it protects devices elsewhere in the circuit.

A Wheatstone bridge circuit is used to measure the resistance of an unknown resistor R.  The bridge ABCD is balanced when X = 2.2 kΩ, Y = 1.0 kΩ and Z = 440 Ω.

- What test would you use to determine that the bridge is balanced?

A galvanometer should be connected between points B and D. When the bridge is balanced, no current will flow through this. If the bridge is unbalanced, a current will flow.

- What is the resistance of the unknown resistor R?

For a balanced bridge,

Rearranging,

- When the unknown resistor R is covered by a piece of black paper, the bridge goes out of balance. What type of resistor is it? Give a use for this type of resistor.

This is obviously a light-dependent resistor (LDR).

They are used in street lights, detecting when the light has gone under a certain level and turning on a light for the night.